Release Date:  05-Sept-2007

- Release Description

This document describes a General Release version 1.1516 of chip firmware and host software for the 57XX-Series SteelVine Storage Processor products.  This release supports SiI5744, SiI5734, SiI5733 and SiI5723 SteelVine Storage Processors in two typical environments.  The first environment uses the product without any management software.  The system is configured using only the chip configuration pins (all status is provided using LED’s).  The second environment is uses the SteelVine Manager GUI in the appropriate OS for interactive configuration and status.

- Applicable New Files

|  | Chip Firmware | SteelVine Manager Host Software for Windows | SteelVine Manager Host Software for Macintosh | SteelVine Manager Host Software for Linux |
| --- | --- | --- | --- | --- |
| File Name: | SiI57x4_11516.bin SiI57x3_11516.bin | 57xxSteelVine_V5.1.20.zip | 57xxSteelVineManager_V5.1.20.zip | 57xxLinuxSteelVineManager_V5.1.20.zip |
| Version: | 1.1516 | 5.1.20 | 5.1.20 | 5.1.20 |
| File Size: | 192 Kbytes | 11,207 Kbytes | 8,527 Kbytes | 9,910 Kbytes |
| File Date: | August 30, 2007 | August 20, 2007 | August 20, 2007 | August 20, 2007 |

- Serial Number and USB ID utility 57xxSN_USB_2.0.4.zip

- Available Product Features

All product features described in the Product Data Sheets and Product Briefs are available in this release except for the following items.

- Manual Drive Locking from user application

- Configuration, monitoring of subordinate nodes in a cascaded configuration through the SteelVine Manager GUI; all subordinate nodes must be configured using the configuration pins[2:0] or directly connected to the root eSATA port individually.

Refer to the 57XX-Series SteelVine Manager User’s Guide for detailed information about the product features.

- Installation Instructions

IMPORTANT NOTE BEFORE INSTALLATION: If you were using any pre-release versions of the SteelVine Manager GUI software, you must use that version to download this new firmware image.  (e.g. update chip firmware in your system before updating GUI).  You must then un-install your existing pre-release SteelVine Manager GUI software before installing this  SteelVine Manager GUI update.  Failure to follow this procedure will cause the 57XX-Series SteelVine Storage Processor to be placed into an unusable state.  Detailed firmware installation and software removal instructions are available in the SiI57XX-Series SteelVine Manager User’s Guide.  To access the User's Guide, select Display Help from the Help menu of the SteelVine Manager GUI.

- RAID Volume Migration from Previous Versions

Any existing RAID volume configurations that were created since the Beta-2 release (dated 21-December-2006) can be used with this firmware version.  Any RAID volumes that were created using any earlier pre-release are not compatible with this firmware version and will not be recognized.

- Qualified Hard Disk Drives

- http://www.siliconimage.com/docs/DOC-002000-451_SteelVineCompatibility_REV_8.pdf

- Qualified SATA Host Adapters

- http://www.siliconimage.com/docs/DOC-002000-451_SteelVineCompatibility_REV_8.pdf

- Qualified SATA-to-PATA Bridge Adapters

JMicron JMB361

SiI3811 (see #3 in Known Issues and Limitations)

- Qualified ATAPI Devices

MSI (XA52P)

Plextor (PX-755SA and PX-7165A)

- Known Issues and Limitations

- In JBOD mode, when connected to a Macintosh SATA host, a slow spin-up inserted device may reset both SATA 57xx ports potentially causing data loss if either drive is being written at the time of reset.  This firmware release is therefore not recommended for external drive enclosures that allow device hotplugs, when connected to Macintosh computers.  External enclosures with fixed drives where power must be removed before replacing a drive will not be affected by this limitation, nor will any external drive that disallows JBOD configurations.  This issue is only seen in JBOD configurations.

- While in GUI mode, replacing both drives of a SAFE set with new drives, at the same time, may cause the GUI to lose communication and force a reset or power cycle to recover proper GUI operation.

- In cascaded configurations where the root node is JBOD, the GUI might display the sub-node configuration on port 0 rather than root-node, due to the bypass nature of the JBOD mode of the root device.

- Firmware should only be downloaded to a single drive in non-cascaded configurations.  Attempting to download firmware when more than one drive is connected may result in both drives being updated or update failure.

- Hosts that are not port multiplier aware (like Intel’s ICH7) will not operate correctly if the 57xx configuration mode is set to JBOD.  GUI will display in-correct information when attempting to use JBOD on a non-PM aware host.  Second drive will be inaccessible.

- ATAPI:

  - ATAPI device support (with GUI) is limited to the following configuration:

    - 57xx mode = JBOD

    - Port 0 – HDD

    - Port 1 – ATAPI device

  - ATAPI device support (without GUI) allows both ports to be used for ATAPI devices.  57xx mode = JBOD

  - ATAPI devices are not supported un DOS mode

  - ATAPI devices are not supported over the USB connection.